# Sensors & Interfacing — Exam Questions

Original exam paper. Answers marked in the source are shown in bold (fill-in answers in blue). Explanations, verifications and corrections are attached as comments on each question heading.

## Question 1 — 3 pts

A periodic signal with a period time of 4s has a frequency in Hz of

0.25

## Question 2 — 3 pts

A periodic signal with a frequency of 2Hz has a period time in seconds of

0.5

## Question 3 — 3 pts

The parameter indicated in the image with the number 3 is:

Group of answer choices

The amplitude

The frequency

The Duty-Cycle

The peak-to-peak value

The pulse width

The period

The sinus

The RMS value

The phase

## Question 4 — 3 pts

A PWM signal is high for 10ms, then remains low for 40ms. The Duty-cycle of this signal is:

Group of answer choices

25ms

10%

100ms

25%

20%

10ms

50ms

40%

## Question 5 — 3 pts

The parameter indicated in the image with the number 1 is:

Group of answer choices

The Duty-Cycle

The phase

The amplitude

The peak-to-peak value

The period

The sinus

The pulse width

The frequency

The RMS value

## Question 6 — 3 pts

A transducer is

Group of answer choices

An element that converts a voltage into a voltage of a different value

Required for analog communication

An element that converts an analog signal or physical parameter into a communication signal

An element that converts a digital signal into an analog signal

An element that converts an analog signal or physical parameter into another analog signal or physical parameter

An element that converts an analog signal into a digital signal

## Question 7 — 3 pts

The following examples of measurable parameters are analog in nature:

Group of answer choices

Visitor counter at a trade fair

Temperature in an oven

The state of a standard Belgian traffic light

Distance to an obstacle measured with an ultrasonic sensor

If a night lamp is turned on

The speed of a bicycle

Air pressure in a car tire

Free space on a hard disk

Precipitation measured in mm/hour

## Question 8 — 3 pts

The parameter indicated in the image with the number 4 is:

Group of answer choices

The amplitude

The frequency

The peak-to-peak value

The pulse width

The Duty-Cycle

The sinus

The period

The RMS value

The phase

## Question 9 — 3 pts

The indicated moment in the signal is:

Group of answer choices

The RMS-time

The duty-cycle

The falling edge

The period time

The rising edge

## Question 10 — 3 pts

A periodic signal with a frequency of 50Hz has a period time in ms of

20

## Question 11 — 3 pts

A sensor

Group of answer choices

Measures a value and makes it digitally readable

Modulates a signal onto a carrier wave

Converts a physical parameter into a signal

Can only measure analog physical values

## Question 12 — 3 pts

A PWM signal is high for 10ms, then remains low for 40ms. The PWM-frequency of this signal is:

Group of answer choices

100Hz

0.05Hz

10ms

20Hz

50ms

20ms

## Question 13 — 3 pts

A periodic signal with a period time of 200ms has a frequency in Hz of

5

## Question 14 — 3 pts

An actuator

Group of answer choices

Converts a carrier wave into a movement of something

Always makes something move based on a digital value

Converts a signal into a physical change

Can only measure analog physical values

Can measure digital values

## Question 15 — 3 pts

The following examples of measurable parameters are digital in nature:

Group of answer choices

The state of a standard Belgian traffic light

If a night lamp is turned on

Distance to an obstacle measured with an ultrasonic sensor

Precipitation measured in mm/hour

The speed of a bicycle

Visitor counter at a trade fair

Air pressure in a car tire

Temperature in an oven

Free space on a hard disk

## Question 16 — 3 pts

The parameter indicated in the image with the number 2 is:

Group of answer choices

The frequency

The sinus

The pulse width

The amplitude

The RMS value

The peak-to-peak value

The Duty-Cycle

The phase

The period

## Question 17 — 3 pts

[The image belonging to this question could not be displayed in the source file.]

The blue signal is a [ Select ] ["AC", "DC"] voltage.   →   marked: DC

The red signal is a [ Select ] ["AC", "DC"] voltage.   →   marked: DC

The green signal is a [ Select ] ["DC", "AC"] voltage.   →   marked: AC

The grey signal is a [ Select ] ["DC", "AC"] voltage.   →   marked: AC

## Question 18 — 3 pts

[The image belonging to this question could not be displayed in the source file.]

The depicted signal is represented in the [ Select ] ["Frequency domain", "Time domain"].

Frequency domain

## Question 19 — 3 pts

You want to digitize a signal while preserving all the information in this signal.

The analog signal has a maximum peak-to-peak value of 3V.

The highest frequency of the signal can be 10 kHz.

What value should be the minimum sample rate in kHz of the A/D converter you have chosen?

20

## Question 20 — 3 pts

The quantization noise of an AD converter can be reduced by

Group of answer choices

Lowering the sample rate

Increasing the signal frequency

Applying aliasing

Adding noise to the analog signal

Removing the noise with a low-pass filter

## Question 21 — 3 pts

The accuracy of an AD converter is determined by

Group of answer choices

Its resolution

Both its range and its resolution

The number of digits in the reading

Its range

## Question 22 — 3 pts

An AD converter has a range between -1.5V and +1.5V, and a resolution of 8 bits.

The value of the LSB is approximately:

Group of answer choices

0.00586V

12mV

0.1176V

1Bit

256Bit

3V

## Question 23 — 3 pts

An AD converter with a range between 0V and 5V and a resolution of 8 bits gives a reading: 0b00000101

The value of the analog input in millivolts (mV) is approximately:

98

## Question 24 — 4 pts

An RS232 connection uses the following communication parameters: 1200-8E2

How long does it take, in seconds, to transmit a data packet of 1200 bytes?

12

## Question 25 — 4 pts

The conversion between a serial and a parallel interface can be done with a:

Group of answer choices

(de-)multiplexer

FIFO-buffer

Shift register

DMA-controller

Transducer

Databus

## Question 26 — 4 pts

An RS232 connection uses the following communication parameters: 1200-8E2

How many bits need to be sent to transmit 1 byte of data?

12

## Question 27 — 4 pts

Simultaneous communication in two directions is called: [ Select ] ["full-duplex", "phased", "I2C", "parallel", "serial", "simplex", "SPI", "half-duplex"] communication

Communication in a single direction is called: [ Select ] ["PSP", "I2C", "phased", "parallel", "serial", "simplex", "full-duplex", "half-duplex"] communication

Bi-directional communication in one direction at a certain moment is called: [ Select ] ["full-duplex", "half-duplex", "I2C", "phased", "parallel", "serial", "SPI", "multimaster", "simplex"] communication

## Question 28 — 4 pts

A MOSI I/O pin on a slave device on a bus is an [ Select ] ["output", "input"].

A MISO I/O pin on a slave device on a bus is an [ Select ] ["input", "output"].

## Question 29 — 4 pts

The I²C bus is [ Select ] ["asynchronous", "synchronous"].

The SPI bus is [ Select ] ["asynchronous", "synchronous"].

## Question 30 — 4 pts

Manchester encoding:

Group of answer choices

Is necessary in I²C communication

Requires a seperate clock signal

Ensures that there are no long periods where the signal level remains the same

Is very suitable for use in wireless ASK connections

## Question 31 — 4 pts

The above image depicts a [ Select ] ["multiplexed", "parallel", "serial", "multimaster"] communication bus.

serial

[Second image could not be displayed in the source file.]

The above image despicts a [ Select ] ["multilayer", "parallel", "multiplexed", "serial"] communication bus.

serial

## Question 32 — 4 pts

An RS232 communication interface has the following parameters: 9600-8N1

How long does it take, approximately, in minutes, to transmit 1MB of data?

18.2

## Question 33 — 4 pts

To increase the noise immunity of a digital communication bus of any length, it's best to:

Group of answer choices

Reduce the voltage

Use differential signals

Lower the frequency

Use a narrowband signal

Increase the frequency

## Question 34 — 4 pts

If an I²C slave cannot process the data from the master quickly enough, then:

Group of answer choices

The master will notice this due to the flow control, and it will send data to the slave at a slower rate

The slave can use clock stretching by pulling the clock line low itself

The slave can send an Xoff Flow-control signal to the master, causing it to temporarily stop sending.

Communication will fail, and the system wil not work

## Question 35 — 4 pts

How many I/O connections are necessary on a controller (master) to connect 5 slaves using an SPI bus? (using the most standard way to implement this bus)

8

## Question 36 — 4 pts

Polling is often necessary in master/slave communication because:

Group of answer choices

It is necessary to know the addresses of the slaves

The slave cannot autonomously send data to the master.

The master might otherwise lose its position in the bus.

The FIFO buffer in the slave might otherwise become full.

## Question 37 — 4 pts

Compare the SPI and I²C interfaces.

In most cases, this interface is faster: [ Select ] ["I²C", "SPI"]

This interface typically consumes less energy in a system: [ Select ] ["I²C", "SPI"]

This interface is easier to implement because bidirectional I/Os are not required: [ Select ] ["I²C", "SPI"]

## Question 38 — 4 pts

In SPI [ Select ] ["Only one master can be present on a bus.", "Potentially, multiple masters can use the same bus"]

In I²C [ Select ] ["Potentially, multiple masters can use the same bus.", "Only one master can be present on a bus."]

## Question 39 — 4 pts

Pull-up resistors are necessary on an SPI bus.

Group of answer choices

True

False

## Question 40 — 4 pts

An I²C bus is always [ Select ] ["full-duplex", "differential", "half-duplex", "integral", "simplex"], while an SPI bus is often [ Select ] ["half-duplex", "differential", "simplex", "full-duplex", "integral"].

marked: half-duplex / full-duplex

## Question 41 — 4 pts

How many I/O connections are necessary on a controller (master) to connect 5 slaves using an I²C bus? (using the most standard way to implement this bus)

2

## Question 42 — 4 pts

Pull-up resistors are necessary on an I²C bus.

Group of answer choices

True

False

## Question 43 — 4 pts

In I²C, the timing is determined by the [ Select ] ["slave", "master"]

In SPi, the timing is determined by the [ Select ] ["slave", "master"]

marked: master / master

## Question 44 — 3 pts

Which modulation technique is the carrier wave depicted below modulated with?

Group of answer choices

ASK

CW

PSK

FSK

FM

AM

## Question 45 — 3 pts

Which analog modulation technique always transmits the same power, regardless of the signal?

Group of answer choices

QAM-256

CPCW

AM

BPSK

FM

## Question 46 — 3 pts

Sending a carrier wave in several different amplitude levels to achieve digital communication is called:

Group of answer choices

OAM-1

ASK

FSK

BPSK

AM

## Question 47 — 3 pts

Which modulation technique for digital signals offers the highest noise immunity among the proposed choises?

(Assuming they are transmitted by a transmitter with equal power and on the same frequency)

Group of answer choices

FSK

BPSK

QAM-256

OFM-2

## Question 48 — 3 pts

Analog SSB modulation is a form of [ Select ] ["CW", "AM", "FM"] where only a single side band is transmitted.

If the part above the carrier is filtered out, we call this [ Select ] ["Firewire", "USB", "LSB"] modulation.

If the part below the carrier is filtered out, we call this [ Select ] ["LSB", "Firewire", "USB"] modulation.

The advantage of SSB modulation is that the bandwidth of the signal is [ Select ] ["decreased", "increased"] and consequently, the required power for the same range is [ Select ] ["increased", "decreased"].

## Question 49 — 3 pts

Sending a carrier wave with a number of different frequency shifts to achieve digital communication is called:

Group of answer choices

OAM

FSK

ASK

BPSK

DTMF

## Question 50 — 3 pts

Which modulation technique for digital signals achieves the highest data rate among the proposed choices?

(Assuming they are transmitted by a transmitter with equal power and on the same frequency)

Group of answer choices

BPSK

QPSK

QAM-256

FSK

CW

ASK

## Question 51 — 3 pts

The specification of a passive antenna states that it has a "gain" of 24dB. It is clear that:

Group of answer choices

This is an omnidirectional antenna.

This specification may not be accurate.

This antenna is an isotropic radiator.

This is a discone antenna.

This is a directional antenna.

## Question 52 — 3 pts

The attenuation of an RF signal in free space without any interfering factors (also known as free-path loss) depends on:

Group of answer choices

Only the frequency.

Only the distance.

The type of modulation, the frequency, and the distance.

Both the frequency of the carrier wave and the distance.

The antenna system and the distance.

The modulation technique and the distance.

## Question 53 — 3 pts

You measure the antenna of a radio receiver. it measures 12.5cm.

You know that this is a classic, non-shortened dipole antenna.

Make an estimate of the frequency for which this receiver is designed. Provide the frequency in MHz.

1.2

## Question 54 — 3 pts

[ Select ] ["Higher", "Lower"] RF frequencies travel further in open space, [ Select ] ["Lower", "Higher"] RF frequencies can penetrate walls better.

## Question 55 — 3 pts

The transmitter of a remote-controlled toy car states that it operates at a frequency of 27MHz.

What is the wavelength of this transmitter in meters?

11.11

## Question 56 — 3 pts

If we safeguard a value of 2 bytes with a CRC, using the CRC polynomial x²+1, how long is the resulting bit string in bits (in total)?

18

## Question 57 — 3 pts

with ECC codes like a CRC, you can secure your system against:

Group of answer choices

Incorrect data during a communication problem such as a collision on the bus

Bit errors caused by disturbances in a cable

Deliberate manipulation of a bitstream

## Question 58 — 3 pts

Complete with the correct parity bit for EVEN parity:

Group of answer choices

1001 0110   [ Choose ] 1 / 0

0101 0100   [ Choose ] 1 / 0

0001 0001   [ Choose ] 1 / 0

1110 1100   [ Choose ] 1 / 0

## Question 59 — 3 pts

A CRC algorithm with a well-chosen polynomial can:

Group of answer choices

Detect 1-bit errors in all cases

Never detect all possible errors in a bitstream

In certain cases even detect errors where more bits are incorrect than the length of its polynomial

## Question 60 — 3 pts

Given a CRC polynomial: x³+x+1

You want to provide a CRC check for the byte: 1110 0011. Add the CRC bits to this data.

The resulting bit string is (provide the full bit pattern including the original bits):

11100011011